389通用说明书

| 4 3 1 2 | 1. ON/OFF Button开关键 2. Mode selector switch模式选择键 3. LCD display LCD屏显示 4. AC indicator 电流提示 |
| --- | --- |
|  | POWER SUPPLY The tools require one 9 volt battery to operate 此工具由9V电池操作 BATTERY INSTALLATION 1. Open battery door on back of case. 2. Connect a 9-volt battery to battery clip. 3. Insert battery into case and replace battery door. 打开电池盖，把电池装好，盖好电池盖 |

Turning the power on/off

On : Press and hold the on/off button on

left side

Off : Release on/off button 开关机，开：按住左边的按键开机，关：放开按键关机

Stud Scan modes

Stud Scan mode for depths to about 3/4

in. (18mm).

Position the Scanner Flat against the

Wall. Select mode. Press and hold on/off     button sounds off the auto finish  Stud指木头支架或者金属支架（一小块的），STUD扫描模式探测深度为18mm， 操作时，将检测仪开放在墙上，按住左侧按键，听到DE的一声后开始移动

Slowly slide unit horizontally across wall, right of left.

When full bars appears on the LCD and steady tone sounds, you have located the edge of the stud. Mark this spot.

Continue holding on/off button in and scan beyond the marked spot until bars disappears  慢慢的水平移动检测仪，从左到右，当LCD显示满的条状体时，说明STUD的边缘测到了，在这个点标注一下，继续同方向移动检测仪，直到显示消失

Without releasing on/off button, slide scanner in reverse direction to locate the other edge of stud. Mark this second spot. Middle of stud is centered between the two marks.不要放开按键，然后反方向移动它，检测另外的边缘，检测到后在这个点标注一下，这两点中间的就是STUD的中心

Calibrating and Scanning in metal

Scan

Select metal mode. Press and hold on/off

button. Then calibrate scanner in air. Away

from metal objects, until the sounds off.

Place scanner flat on the surface.金属检测的

校正和扫描，选择METAL模式，按住左侧按键

在空气中校正，检测到后离开物体到声音消失

While holding in on/off button, slowly move

unit randomly to locate target LCD screen

indicates proximity to metal, with center bar indicating closest proximity. Mark this spot.

按住左侧按键，慢慢在墙上移动检测仪来定位金属位置，显示中心条状体说明检测到了，在这点上标注。

Continue scanning beyond the marked spot to detect width of target. If target is wide , reverse to locate and mark the other edge. Avoid sawing or drilling between the two marks. Widths will rebar to wide area for metal studs, I beams , junction boxes. 继续移动来检测金属宽度，如果目标很宽，反方向检测，找到另一边缘时标注，避免在此打孔打钻。

Scanning in AC mode

AC depths to about 2 in.

Select AC scan mode to determine position

of hot wires. Begin by placing the scanner

at some position away from hot wiring.

While holding in the on/off button, slowly slide unit across the wall. AC indicator will appear on LCD if a hot wire is nearby. LCD display indicates proximity to hot wire all bars indicating closest proximity. Mark this mark. The scanner detects from 90V to 250V at 50 to 60 Hz AC in a HOT electrical wire. The voltage detection feature works continuously in all modes for safety 电流检测：选择AC模式来检测电线，先在离电线一定距离的位置然后按住按键，慢慢靠近电线，电线有电时，LCD屏会显示也会叫,LCD屏满条状体时说明电线已经检测到，在此标注，测电线AC从90V到250V，50到60Hz AC交流。电压检测在所有模式里是安全的。

Auto Scan Mode

When select “auto mode” the detector will search for three objects metal, cable and stud at the same time and indicate what has been detected in it’s LCD display, Press and hold the on/off button on left side, slowly slide unit horizontally. If two or three different objects are found simultaneously both will be indicated (the one with the strongest signal icon will display always, the others icon will display flash)

自动检测模式

当选择自动模式时，检测仪会同时自动搜索三种检测目标，metal, cable，stud，并会在LCD显示屏上显示。按住左边按键，慢慢水平移动，如果两个或三个目标同时发现，就会提示出来，（信号强时显示信号强的物体，其次的会显示闪烁）

.